#include <Wire.h>

// MPU-6050 I2C address

const int MPU_addr = 0x68;

// Gyroscope and accelerometer vars

int16_t AcX, AcY, AcZ, GyX, GyY, GyZ;

float gyroX, gyroY, gyroZ;  // Angular velocity in °/s

float accX, accY, accZ;     // Accelerometer data in g

// Orientation variables

float roll, pitch, yaw;      // Roll, pitch, and yaw angles

//Note: Dependent on how the sensor is place affects these (roll and pitch are switched currently)

float rollAcc, pitchAcc;     // Roll and pitch from accelerometer

// Complementary filter variables

float alpha = 0.98;  // Weight for gyroscope data (tune this value if needed)

// Timing variables

float dt;                   // Time interval

unsigned long prevTime;     // Previous time stamp

// Gyroscope sensitivity scale factor (for ±250°/s range)

const float GYRO_SCALE = 131.0;

// Accelerometer sensitivity scale factor (for ±2g range)

const float ACCEL_SCALE = 16384.0;

void setup() {

Wire.begin();

Serial.begin(9600);

// Initialize MPU-6050

Wire.beginTransmission(MPU_addr);

Wire.write(0x6B);  // PWR_MGMT_1 register

Wire.write(0);     // Wake up the MPU-6050

Wire.endTransmission(true);

// Initialize variables

roll = 0;

pitch = 0;

yaw = 0;

prevTime = micros();

}

void loop() {

// Read accelerometer and gyroscope data

Wire.beginTransmission(MPU_addr);

Wire.write(0x3B);  // Start with register 0x3B (ACCEL_XOUT_H)

Wire.endTransmission(false);

Wire.requestFrom(MPU_addr, 14, true);  // Request 14 registers

// Read accelerometer data

AcX = Wire.read() << 8 | Wire.read();

AcY = Wire.read() << 8 | Wire.read();

AcZ = Wire.read() << 8 | Wire.read();

// Read temperature data (not used in this example)

Wire.read() << 8 | Wire.read();

// Read gyroscope data

GyX = Wire.read() << 8 | Wire.read();

GyY = Wire.read() << 8 | Wire.read();

GyZ = Wire.read() << 8 | Wire.read();

// Convert raw accelerometer data to g

accX = AcX / ACCEL_SCALE;

accY = AcY / ACCEL_SCALE;

accZ = AcZ / ACCEL_SCALE;

// Convert raw gyroscope data to °/s

gyroX = GyX / GYRO_SCALE;

gyroY = GyY / GYRO_SCALE;

gyroZ = GyZ / GYRO_SCALE;

// Calculate time interval

unsigned long currentTime = micros();

dt = (currentTime - prevTime) / 1000000.0;  // Convert to seconds

prevTime = currentTime;

// Calculate roll and pitch from accelerometer

rollAcc = atan2(accY, accZ) * 180 / PI;

pitchAcc = atan2(-accX, sqrt(accY * accY + accZ * accZ)) * 180 / PI;

// Apply complementary filter to combine gyroscope and accelerometer data

roll = alpha * (roll + gyroX * dt) + (1 - alpha) * rollAcc;

pitch = alpha * (pitch + gyroY * dt) + (1 - alpha) * pitchAcc;

yaw += gyroZ * dt;  // Yaw is only from gyroscope (no accelerometer correction)

// Print orientation

Serial.print("Pitch: ");

Serial.print(roll);

Serial.print(" | Roll: ");

Serial.print(pitch);

Serial.print(" | Yaw: ");

Serial.println(yaw);

delay(10);  // Small delay for stability

}