#include <Wire.h>

#include "Adafruit_VL53L0X.h"

// Define sensor objects with position labels

Adafruit_VL53L0X loxFarRight = Adafruit_VL53L0X(); // Pin 4

Adafruit_VL53L0X loxMidRight = Adafruit_VL53L0X();  // Pin 5

Adafruit_VL53L0X loxMidLeft = Adafruit_VL53L0X();   // Pin 6

Adafruit_VL53L0X loxFarLeft = Adafruit_VL53L0X();   // Pin 7

// Define XSHUT pins for each position

#define XSHUT_FAR_RIGHT 4

#define XSHUT_MID_RIGHT 5

#define XSHUT_MID_LEFT 6

#define XSHUT_FAR_LEFT 7

void setup() {

Serial.begin(115200);

Wire.begin(); // Initialize I2C

// Set XSHUT pins as outputs

pinMode(XSHUT_FAR_RIGHT, OUTPUT);

pinMode(XSHUT_MID_RIGHT, OUTPUT);

pinMode(XSHUT_MID_LEFT, OUTPUT);

pinMode(XSHUT_FAR_LEFT, OUTPUT);

// Initialize sensors in order (prevents address conflicts)

initializeSensor(loxFarRight, XSHUT_FAR_RIGHT, 0x30, "Far Right");

initializeSensor(loxMidRight, XSHUT_MID_RIGHT, 0x31, "Middle Right");

initializeSensor(loxMidLeft, XSHUT_MID_LEFT, 0x32, "Middle Left");

initializeSensor(loxFarLeft, XSHUT_FAR_LEFT, 0x33, "Far Left");

Serial.println(F("All sensors initialized!"));

}

void loop() {

// Read and print distances with position labels

readSensor(loxFarRight, "Far Right");

readSensor(loxMidRight, "Middle Right");

readSensor(loxMidLeft, "Middle Left");

readSensor(loxFarLeft, "Far Left");

delay(1000); // Adjust for robot responsiveness

}

// Function to initialize a sensor with a unique address and label

void initializeSensor(Adafruit_VL53L0X &sensor, int xshutPin, uint8_t newAddress, const char* position) {

// Reset the sensor (pull XSHUT low, then high)

digitalWrite(xshutPin, LOW);

delay(10);

digitalWrite(xshutPin, HIGH);

delay(10);

// Initialize with the new address

if (!sensor.begin(newAddress)) {

Serial.print(position);

Serial.println(F(" sensor failed to boot!"));

while (1); // Halt if a sensor fails

}

Serial.print(position);

Serial.print(F(" sensor initialized at address 0x"));

Serial.println(newAddress, HEX);

}

// Function to read and print sensor data

void readSensor(Adafruit_VL53L0X &sensor, const char* position) {

VL53L0X_RangingMeasurementData_t measure;

sensor.rangingTest(&measure, false); // Get distance

if (measure.RangeStatus != 4) {

Serial.print(position);

Serial.print(": ");

Serial.print(measure.RangeMilliMeter);

Serial.println(" mm");

} else {

Serial.print(position);

Serial.println(": Out of range");

}

}