#include <Wire.h>

#include <Adafruit_VL53L0X.h>

/*************************** MPU-9250/6500 Configuration ***************************/

/*

MPU-9250/6500 Connections

SDA -> SDA

SCL -> SCL

*/

const int MPU_ADDR = 0x68;

const float GYRO_SCALE = 131.0; // ±250°/s range

float yaw = 0;

float gyroZOffset = 0;

unsigned long prevTime = 0;

const unsigned long yawPrintInterval = 1000; // Print every 1000 ms

unsigned long lastYawPrint = 0;            // Timing tracker

/******************************* ToF Sensors Setup ********************************/

Adafruit_VL53L0X loxFarRight = Adafruit_VL53L0X(); // Pin 4

Adafruit_VL53L0X loxMidRight = Adafruit_VL53L0X();  // Pin 5

Adafruit_VL53L0X loxMidLeft = Adafruit_VL53L0X();   // Pin 6

Adafruit_VL53L0X loxFarLeft = Adafruit_VL53L0X();   // Pin 7

#define XSHUT_FAR_RIGHT 4

#define XSHUT_MID_RIGHT 5

#define XSHUT_MID_LEFT 6

#define XSHUT_FAR_LEFT 7

unsigned long lastToFRead = 0;

/***************************** Sound & Motion Sensors *****************************/

const int sensorAnalogPin = A0;     // Analog input

const int sensorDigitalPin = 28;    // Digital input D28

const int ledPin = LED_BUILTIN;     // Built-in LED

// Sound detection settings

unsigned long lastNoiseTime = 0;

const unsigned long noiseCooldown = 1000;

int loudNoiseThreshold;

int quietLevel;

// PIR Sensor Pins (Inputs)

int pirPin1 = 22;  // HC-SR501 #1

int pirPin2 = 23;  // HC-SR501 #2

int pirPin3 = 24;  // HC-SR501 #3

int pirPin4 = 25;  // HC-SR501 #4

// Motion detection settings

unsigned long lastMotionTime = 0;

const unsigned long motionCooldown = 1000;

/********************************** Core Functions ********************************/

void setup() {

Serial.begin(115200);

Wire.begin();

// Initialize MPU

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B); // PWR_MGMT_1

Wire.write(0);    // Wake up

Wire.endTransmission();

calibrateGyro();

// Initialize ToF sensors

pinMode(XSHUT_FAR_RIGHT, OUTPUT);

pinMode(XSHUT_MID_RIGHT, OUTPUT);

pinMode(XSHUT_MID_LEFT, OUTPUT);

pinMode(XSHUT_FAR_LEFT, OUTPUT);

initializeSensor(loxFarRight, XSHUT_FAR_RIGHT, 0x30, "Far Right");

initializeSensor(loxMidRight, XSHUT_MID_RIGHT, 0x31, "Middle Right");

initializeSensor(loxMidLeft, XSHUT_MID_LEFT, 0x32, "Middle Left");

initializeSensor(loxFarLeft, XSHUT_FAR_LEFT, 0x33, "Far Left");

// Initialize sound/motion sensors

pinMode(sensorDigitalPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(pirPin1, INPUT);

pinMode(pirPin2, INPUT);

pinMode(pirPin3, INPUT);

pinMode(pirPin4, INPUT);

calibrateSensor();

Serial.println("System initialization complete!");

}

void loop() {

// Update orientation data

updateGyro();

// Handle yaw printing at controlled rate

printYawIfNeeded();

// Handle ToF sensor readings

if (millis() - lastToFRead >= 1000) {

readAllToFSensors();

lastToFRead = millis();

}

// Monitor environment

checkSound();

checkMotion();

}

/********************************** MPU Functions *********************************/

void calibrateGyro() {

Serial.println("Calibrating gyro... Keep sensor stationary!");

delay(1000);

long total = 0;

for(int i = 0; i < 1000; i++) {

total += readGyroZ();

delay(1);

}

gyroZOffset = total / 1000.0;

Serial.print("Calibration offset: ");

Serial.println(gyroZOffset);

}

int16_t readGyroZ() {

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x47); // GYRO_ZOUT_H

Wire.endTransmission(false);

Wire.requestFrom(MPU_ADDR, 2, true);

return Wire.read() << 8 | Wire.read();

}

// Modified: Removed Serial.print from updateGyro

void updateGyro() {

int16_t rawGyroZ = readGyroZ();

float gyroZ = (rawGyroZ - gyroZOffset) / GYRO_SCALE;

unsigned long currentTime = micros();

float dt = (currentTime - prevTime) / 1000000.0;

prevTime = currentTime;

yaw += gyroZ * dt;

}

// New: Dedicated yaw printing function

void printYawIfNeeded() {

if (millis() - lastYawPrint >= yawPrintInterval) {

Serial.print("Yaw: ");

Serial.println(yaw, 2);

lastYawPrint = millis();

}

}

/********************************* ToF Functions *********************************/

void initializeSensor(Adafruit_VL53L0X &sensor, int xshutPin, uint8_t newAddress, const char* position) {

digitalWrite(xshutPin, LOW);

delay(10);

digitalWrite(xshutPin, HIGH);

delay(10);

if (!sensor.begin(newAddress)) {

Serial.print(position);

Serial.println(" sensor failed to boot!");

while(1);

}

Serial.print(position);

Serial.print(" sensor initialized at 0x");

Serial.println(newAddress, HEX);

}

void readSensor(Adafruit_VL53L0X &sensor, const char* position) {

VL53L0X_RangingMeasurementData_t measure;

sensor.rangingTest(&measure, false);

if(measure.RangeStatus != 4) {

Serial.print(position);

Serial.print(": ");

Serial.print(measure.RangeMilliMeter);

Serial.println(" mm");

} else {

Serial.print(position);

Serial.println(": Out of range");

}

}

void readAllToFSensors() {

readSensor(loxFarRight, "Far Right");

readSensor(loxMidRight, "Middle Right");

readSensor(loxMidLeft, "Middle Left");

readSensor(loxFarLeft, "Far Left");

}

/****************************** Sound/Motion Functions ****************************/

void calibrateSensor() {

long sum = 0;

for(int i = 0; i < 50; i++) sum += analogRead(sensorAnalogPin);

quietLevel = sum / 50;

loudNoiseThreshold = 400; // Hard-coded for testing

Serial.println("Sound sensor calibrated");

}

void checkSound() {

int analogValue = analogRead(sensorAnalogPin);

int digitalValue = digitalRead(sensorDigitalPin);

if((analogValue > loudNoiseThreshold) && (digitalValue == HIGH) &&

(millis() - lastNoiseTime > noiseCooldown)) {

handleNoiseDetection(analogValue);

triggerActionSound();

}

}

void handleNoiseDetection(int noiseLevel) {

lastNoiseTime = millis();

Serial.print("LOUD NOISE! Level: ");

Serial.println(noiseLevel);

for(int i = 0; i < 3; i++) {

digitalWrite(ledPin, HIGH);

delay(100);

digitalWrite(ledPin, LOW);

delay(100);

}

}

void triggerActionSound() {

Serial.println("Action: Sound detected!");

}

void checkMotion() {

if((digitalRead(pirPin1) || digitalRead(pirPin2) ||

digitalRead(pirPin3) || digitalRead(pirPin4)) &&

(millis() - lastMotionTime > motionCooldown)) {

handleMotionDetection();

}

}

void handleMotionDetection() {

lastMotionTime = millis();

Serial.println("Motion detected");

}