const int leftencoderApin = 2;

const int leftencoderBpin = 3;

const int rightencoderApin = 18;

const int rightencoderBpin = 19;

const int leftdirpin = 11;

const int rightdirpin = 12;

const int leftpwmpin = 9;

const int rightpwmpin = 10;

int leftencoderpulse = 0; // ticks left

int rightencoderpulse = 0; // ticks right

// distance variables

double ipr = 18.85; // inches per revolution

double tpr = 950; // ticks per revolution

// PID variables

int pwm = 100; // pwm motor power

double error = 0; // set speed - actual speed

double eI = 0; // integral error

double eD = 0;  // derivative error

double prev_error = 0;

const float Kp = 1.0;  // proportional gain

const float Ki = 2.0;  //integral gain

const float Kd = 1.0;  // derivative gain

float targetAngle = 90;

int i = 0; // for motor start delay

// Odometry variables

const float wheelbase = 13.7; // inches

const float dpt = ipr / tpr;

float dL = 0; // distance left wheel traveled

float dR = 0; // distance right wheel traveled

float theta = 0;

void setup()

{

Serial.begin(9600);

pinMode(leftencoderApin, INPUT); //input_pullup to print to serial monitor..?

pinMode(leftencoderBpin, INPUT);

// Left encoder interrupts

attachInterrupt(digitalPinToInterrupt(leftencoderApin), ALeadingLeft, RISING);

attachInterrupt(digitalPinToInterrupt(leftencoderBpin), BLeadingLeft, RISING);

// Right encoder interrupts

attachInterrupt(digitalPinToInterrupt(rightencoderApin), ALeadingRight, RISING);

attachInterrupt(digitalPinToInterrupt(rightencoderBpin), BLeadingRight, RISING);

// target ticks

//targetticks = (targetdistance / ipr) * tpr;

pinMode(leftdirpin, OUTPUT);

pinMode(rightdirpin, OUTPUT);

pinMode(leftpwmpin, OUTPUT);

pinMode(rightpwmpin, OUTPUT);

analogWrite(leftpwmpin, 0);

analogWrite(rightpwmpin, 0);

delay(2000); // Allows the serial monitor to connect before robot starts

}

void loop()

{

// clockwise forward motion (hopefully)

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

//unsigned long clock = millis();

//computePID();

//analogWrite(leftpwmpin, 255-pwm-45);

// Serial.println("before");

// Serial.println(i);

// // start delay on motors

// if(i < 1)

// {

//   Serial.println("during");

//   delay(2000);

//   Serial.print(i);

//   i++;

// }

// Serial.println("after");

// Serial.println(i);

//analogWrite(rightpwmpin, pwm);

changeAngle();

Serial.print("dl: ");

Serial.println(dL);

Serial.print("dr: ");

Serial.println(dR);

Serial.print("theta: ");

Serial.println(theta);

Serial.print("error: ");

Serial.println(error);

//Serial.print("left encoder pulse: ");

// Serial.println(leftencoderpulse);

// Serial.print(",");

// //Serial.print("right encoder pulse: ");

// Serial.println(rightencoderpulse);

delay(50);

// Serial.print("Distance Traveled: ");

// Serial.print((((leftencoderpulse + rightencoderpulse) / 2) / tpr) * ipr);

// Serial.println(" inches");

while(((leftencoderpulse + rightencoderpulse) / 2) >= tpr)

{

stopMotors();

//Serial.println("ENCODER STOPPED");

}

}

void stopMotors()

{

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 255);

analogWrite(rightpwmpin, 0);

}

void ALeadingLeft()

{

if(digitalRead(leftencoderBpin) == LOW)

{

leftencoderpulse--;

}

else

{

leftencoderpulse++;

}

}

void BLeadingLeft()

{

if(digitalRead(leftencoderApin) == LOW)

{

leftencoderpulse++;

}

else

{

leftencoderpulse--;

}

}

void ALeadingRight()

{

if(digitalRead(rightencoderBpin) == LOW)

{

rightencoderpulse++;

}

else

{

rightencoderpulse--;

}

}

void BLeadingRight()

{

if(digitalRead(rightencoderApin) == LOW)

{

rightencoderpulse--;

}

else

{

rightencoderpulse++;

}

}

void computePID()

{

error = targetAngle - theta;

eI = eI + error; // integral error

eD = prev_error - error; // derivative error

prev_error = error;

// limit overflow of intergral error

if (eI > 200)

{

eI = 0;

}

pwm = Kp * error + Ki * eI + Kd * eD; // P + I + D

if (pwm >= 360) // limit max

{

}

else if (pwm <= -360) // limit min

{

}

}

void computeAngle()

{

dL = leftencoderpulse * dpt;

dR = rightencoderpulse * dpt;

theta = (dL-dR)/wheelbase * (180/3.1415);

}

void changeAngle()

{

computeAngle();

error = targetAngle - theta;

if (error > 5) // turn left

{

digitalWrite(leftdirpin, LOW);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 100+45);

analogWrite(rightpwmpin, 100);

} else if (error < -5) // this is the one we want to happen, turn right

{

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, HIGH);

analogWrite(leftpwmpin, 255-100-45);

analogWrite(rightpwmpin, 255-100);

}

else // default

{

stopMotors();

// digitalWrite(leftdirpin, HIGH);

// digitalWrite(rightdirpin, LOW);

// analogWrite(leftpwmpin, 255-100-45);

// analogWrite(rightpwmpin, 100);

delay(3000);

}

}