#include <Wire.h>

#include <Adafruit_VL53L0X.h>

#include <IRremote.h>

const int leftencoderApin = 2;

const int leftencoderBpin = 3;

const int rightencoderApin = 18;

const int rightencoderBpin = 19;

const int leftdirpin = 11;

const int rightdirpin = 12;

const int leftpwmpin = 9;

const int rightpwmpin = 10;

int leftencoderpulse = 0;

int rightencoderpulse = 0;

int currentleftencoderpulse = 0;

int currentrightencoderpulse = 0;

// distance variables

double ipr = 18.85; // inches per revolution

double tpr = 900; // ticks per revolution

int targetangle = 0;

int pwm = 100; // pwm motor power

int offset = 25; // motor power offset

// PID variables

double error = 0; // set speed - actual speed

double eI = 0; // integral error

double eD = 0;  // derivative error

double prev_error = 0;

const float Kp = 1.8;  // proportional gain

const float Ki = 0.4;  //integral gain

const float Kd = 0.05;  // derivative gain

float omega = 0;

float vL = 0;

float vR = 0;

int i = 0; // for motor start delay

const float wheelbase = 13.7; // inches

bool runonceturn = false; // the run once bool for turning

// Gyro Variables

const int MPU_ADDR = 0x68;

const float GYRO_SCALE = 131.0; // ±250°/s range

float yaw = 0;

float gyroZOffset = 0;

unsigned long prevTime = 0;

// ToF Variables

Adafruit_VL53L0X loxFarRight = Adafruit_VL53L0X(); // Pin 4

Adafruit_VL53L0X loxMidRight = Adafruit_VL53L0X();  // Pin 5

Adafruit_VL53L0X loxMidLeft = Adafruit_VL53L0X();   // Pin 6

Adafruit_VL53L0X loxFarLeft = Adafruit_VL53L0X();   // Pin 7

#define XSHUT_FAR_RIGHT 4

#define XSHUT_MID_RIGHT 5

#define XSHUT_MID_LEFT 6

#define XSHUT_FAR_LEFT 7

unsigned long lastToFRead = 0;

float distanceFarRight = 0;

float distanceMidRight = 0;

float distanceMidLeft = 0;

float distanceFarLeft = 0;

float distancethreshold = 200;

bool farRight = (distanceFarRight <= distancethreshold);

bool midRight = (distanceMidRight <= distancethreshold);

bool midLeft = (distanceMidLeft <= distancethreshold);

bool farLeft = (distanceFarLeft <= distancethreshold);

// Sound and Motion Variables

const int sensorAnalogPin = A0;     // Analog input

const int sensorDigitalPin = 28;    // Digital input D28

const int ledPin = LED_BUILTIN;     // Built-in LED

// Sound detection settings

unsigned long lastNoiseTime = 0;

const unsigned long noiseCooldown = 1000;

int loudNoiseThreshold;

int quietLevel;

// PIR Sensor Pins (Inputs)

int pirPin1 = 22;  // HC-SR501 #1

int pirPin2 = 23;  // HC-SR501 #2

int pirPin3 = 24;  // HC-SR501 #3

int pirPin4 = 25;  // HC-SR501 #4

// Motion detection settings

unsigned long lastMotionTime = 0;

const unsigned long motionCooldown = 1000;

// IR Configuration

const int receiver = 8;       // IR receiver pin

IRrecv irrecv(receiver);      // Create IR receiver object

static uint32_t last_decodedRawData = 0;  // Stores last received code

// State management

enum SystemState

{

OFF,

STANDBY,

MOVING,

SEARCHING

};

SystemState currentState = OFF;

/***************************** Start of Setup *****************************/

void setup()

{

Serial.begin(115200);

Wire.begin();

pinMode(leftencoderApin, INPUT);

pinMode(leftencoderBpin, INPUT);

// Left encoder interrupts

attachInterrupt(digitalPinToInterrupt(leftencoderApin), ALeadingLeft, RISING);

attachInterrupt(digitalPinToInterrupt(leftencoderBpin), BLeadingLeft, RISING);

// Right encoder interrupts

attachInterrupt(digitalPinToInterrupt(rightencoderApin), ALeadingRight, RISING);

attachInterrupt(digitalPinToInterrupt(rightencoderBpin), BLeadingRight, RISING);

pinMode(leftdirpin, OUTPUT);

pinMode(rightdirpin, OUTPUT);

pinMode(leftpwmpin, OUTPUT);

pinMode(rightpwmpin, OUTPUT);

analogWrite(leftpwmpin, 0);

analogWrite(rightpwmpin, 0);

// Gyro setup (Initialize MPU)

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x6B); // PWR_MGMT_1

Wire.write(0);     // Wake up

Wire.endTransmission();

calibrateGyro();

prevTime = micros();

// IR initialization

irrecv.enableIRIn();  // Start the IR receiver

// Initialize ToF sensors

pinMode(XSHUT_FAR_RIGHT, OUTPUT);

pinMode(XSHUT_MID_RIGHT, OUTPUT);

pinMode(XSHUT_MID_LEFT, OUTPUT);

pinMode(XSHUT_FAR_LEFT, OUTPUT);

initializeSensor(loxFarRight, XSHUT_FAR_RIGHT, 0x30, "Far Right");

initializeSensor(loxMidRight, XSHUT_MID_RIGHT, 0x31, "Middle Right");

initializeSensor(loxMidLeft, XSHUT_MID_LEFT, 0x32, "Middle Left");

initializeSensor(loxFarLeft, XSHUT_FAR_LEFT, 0x33, "Far Left");

// Initialize sound/motion sensors

pinMode(sensorDigitalPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(pirPin1, INPUT);

pinMode(pirPin2, INPUT);

pinMode(pirPin3, INPUT);

pinMode(pirPin4, INPUT);

calibrateSensor();

Serial.println("System initialization complete!");

delay(2000); // Allows the serial monitor to connect before robot starts

}

void loop()

{

readAllToFSensors();

if(checkFrontObstacle())

{

double randomTurn = random(100, 260);

Serial.println("Object Detected");

readAllToFSensors();

runonceturn = true;

stopMotors();

targetangle = yaw + randomTurn;

delay(1000);

while(runonceturn == true)

{

updateGyro();

Serial.println(yaw);

Serial.println(targetangle);

Serial.println("TURNING");

computePID();

turnRight();

Serial.println(runonceturn);

}

} else if ((leftencoderpulse < (currentleftencoderpulse + 60*(tpr/ipr))) && (rightencoderpulse < (currentrightencoderpulse + 60*(tpr/ipr))))// left encoder pulse and right in coder pulse less than current left encoder pulse plus 5ft and current right encoder pulse pluse 5ft

{

Serial.println("normal case ");

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 255-pwm-offset); // OFFSET LINE

analogWrite(rightpwmpin, pwm);

updateGyro();

} else

{

stopMotors();

delay(3000);

// Check for IR input

if (irrecv.decode())

{

translateIR();

irrecv.resume();

}

currentleftencoderpulse = leftencoderpulse;

currentrightencoderpulse = rightencoderpulse;

}

// Handle ToF sensor readings

if (millis() - lastToFRead >= 1000) {

readAllToFSensors();

lastToFRead = millis();

}

}

void stopMotors()

{

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 255);

analogWrite(rightpwmpin, 0);

}

void ALeadingLeft()

{

if(digitalRead(leftencoderBpin) == LOW)

{

leftencoderpulse--;

}

else

{

leftencoderpulse++;

}

}

void BLeadingLeft()

{

if(digitalRead(leftencoderApin) == LOW)

{

leftencoderpulse++;

}

else

{

leftencoderpulse--;

}

}

void ALeadingRight()

{

if(digitalRead(rightencoderBpin) == LOW)

{

rightencoderpulse++;

}

else

{

rightencoderpulse--;

}

}

void BLeadingRight()

{

if(digitalRead(rightencoderApin) == LOW)

{

rightencoderpulse--;

}

else

{

rightencoderpulse++;

}

}

void computePID()

{

error = targetangle - yaw;

eI = eI + error; // integral error

eD = prev_error - error; // derivative error

prev_error = error;

// limit overflow of intergral error

if (eI > 200)

{

eI = 0;

}

omega = Kp * error + Ki * eI + Kd * eD; // calculating rate of change of angle (rad/s)

if (omega >= 125) // limit max

{

omega = 125;

}

else if (omega <= 0) // limit min

{

omega = 80;

}

}

void turnRight()

{

vL = ((omega * wheelbase) / 2);

vR = ((omega * wheelbase) / 2);

if(targetangle > yaw)

{

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, HIGH);

analogWrite(leftpwmpin, 255-(vL*0.152)-45); // OFFSET LINE

analogWrite(rightpwmpin, 255-(vR*0.152));

} else

{

stopMotors();

runonceturn = false;

}

}

int16_t readGyroZ()

{

Wire.beginTransmission(MPU_ADDR);

Wire.write(0x47); // GYRO_ZOUT_H

Wire.endTransmission(false);

Wire.requestFrom(MPU_ADDR, 2, true);

return Wire.read() << 8 | Wire.read();

}

void calibrateGyro()

{

Serial.println("Calibrating gyro... Keep sensor stationary!");

delay(1000);

long total = 0;

for(int i = 0; i < 1000; i++)

{

total += readGyroZ();

delay(1);

}

gyroZOffset = total / 1000.0;

Serial.print("Calibration offset: ");

Serial.println(gyroZOffset);

}

void updateGyro()

{

int16_t rawGyroZ = readGyroZ();

float gyroZ = (rawGyroZ - gyroZOffset) / GYRO_SCALE;

unsigned long currentTime = micros();

float dt = (currentTime - prevTime) / 1000000.0;

prevTime = currentTime;

yaw -= gyroZ * dt;

}

/********************************* ToF Functions *********************************/

void initializeSensor(Adafruit_VL53L0X &sensor, int xshutPin, uint8_t newAddress, const char* position)

{

digitalWrite(xshutPin, LOW);

delay(10);

digitalWrite(xshutPin, HIGH);

delay(10);

if (!sensor.begin(newAddress))

{

Serial.print(position);

Serial.println(" sensor failed to boot!");

while(1);

}

Serial.print(position);

Serial.print(" sensor initialized at 0x");

Serial.println(newAddress, HEX);

}

int readSensor(Adafruit_VL53L0X &sensor, const char* position)

{

VL53L0X_RangingMeasurementData_t measure;

sensor.rangingTest(&measure, false);

if(measure.RangeStatus != 4)

{

Serial.print(position);

Serial.print(": ");

Serial.print(measure.RangeMilliMeter);

Serial.println(" mm");

return(measure.RangeMilliMeter);

} else

{

Serial.print(position);

Serial.println(": Out of range");

return(-1);

}

}

void readAllToFSensors()

{

distanceFarRight = readSensor(loxFarRight, "Far Right");

distanceMidRight = readSensor(loxMidRight, "Middle Right");

distanceMidLeft = readSensor(loxMidLeft, "Middle Left");

distanceFarLeft = readSensor(loxFarLeft, "Far Left");

}

bool checkFrontObstacle()

{

VL53L0X_RangingMeasurementData_t measure;

bool obstacleDetected = false;

// Check first sensor

loxFarLeft.rangingTest(&measure, false);

obstacleDetected |= (measure.RangeMilliMeter <= 400 && measure.RangeStatus != 4);

// Check second sensor

loxMidLeft.rangingTest(&measure, false);

obstacleDetected |= (measure.RangeMilliMeter <= 400 && measure.RangeStatus != 4);

// Check third sensor

loxMidRight.rangingTest(&measure, false);

obstacleDetected |= (measure.RangeMilliMeter <= 400 && measure.RangeStatus != 4);

// Check fourth sensor

loxFarRight.rangingTest(&measure, false);

obstacleDetected |= (measure.RangeMilliMeter <= 400 && measure.RangeStatus != 4);

return obstacleDetected;

}

/****************************** Sound/Motion Functions ****************************/

void calibrateSensor()

{

long sum = 0;

for(int i = 0; i < 50; i++) sum += analogRead(sensorAnalogPin);

quietLevel = sum / 50;

loudNoiseThreshold = 400; // Hard-coded for testing

Serial.println("Sound sensor calibrated");

}

void checkSound()

{

int analogValue = analogRead(sensorAnalogPin);

int digitalValue = digitalRead(sensorDigitalPin);

if((analogValue > loudNoiseThreshold) && (digitalValue == HIGH) &&

(millis() - lastNoiseTime > noiseCooldown)) {

handleNoiseDetection(analogValue);

triggerActionSound();

}

}

void handleNoiseDetection(int noiseLevel)

{

lastNoiseTime = millis();

Serial.print("LOUD NOISE! Level: ");

Serial.println(noiseLevel);

for(int i = 0; i < 3; i++)

{

digitalWrite(ledPin, HIGH);

delay(100);

digitalWrite(ledPin, LOW);

delay(100);

}

}

void triggerActionSound()

{

Serial.println("Action: Sound detected!");

}

void checkMotion()

{

if((digitalRead(pirPin1) || digitalRead(pirPin2) ||

digitalRead(pirPin3) || digitalRead(pirPin4)) &&

(millis() - lastMotionTime > motionCooldown))

{

handleMotionDetection();

}

}

void handleMotionDetection()

{

lastMotionTime = millis();

Serial.println("Motion detected");

}

/****************************** Remote/IR Functions ****************************/

void translateIR()

{

if (irrecv.decode())

{

static unsigned long lastIRTime = 0;

unsigned long currentTime = millis();

// Debounce - ignore repeats within 200ms

if (currentTime - lastIRTime > 200)

{

lastIRTime = currentTime;

switch(irrecv.decodedIRData.decodedRawData)

{

case 0xBA45FF00: // POWER button

if (currentState == OFF)

{

currentState = STANDBY;

} else if (currentState == STANDBY)

{

currentState = MOVING;

} else

{

currentState = OFF;

}

break;

case 0xB847FF00: // RESET button

currentState = STANDBY;

break;

}

}

}

}