#include <Wire.h>

#include <Adafruit_VL53L0X.h>

#include <IRremote.h>

/******************************* Pin Definitions ********************************/

// Function prototypes

void calibrateGyro();

void updateGyro();

int16_t readGyroZ();

// ToF XSHUT Pins

#define XSHUT_FAR_RIGHT 4

#define XSHUT_MID_RIGHT 5

#define XSHUT_MID_LEFT 6

#define XSHUT_FAR_LEFT 7

// Motor Control

const int leftencoderApin = 2;

const int leftencoderBpin = 3;

const int rightencoderApin = 18;

const int rightencoderBpin = 19;

const int leftdirpin = 11;

const int rightdirpin = 12;

const int leftpwmpin = 9;

const int rightpwmpin = 10;

// IR & LEDs

#define RED 30

#define GREEN 31

#define BLUE 32

const int receiver = 8; // IR receiver pin

// Sound & Motion

const int sensorAnalogPin = A0;

const int sensorDigitalPin = 28;

const int ledPin = LED_BUILTIN;

const int pirPin1 = 22;

const int pirPin2 = 23;

const int pirPin3 = 24;

const int pirPin4 = 25;

const int buzzerPin = 29;         // Buzzer connected to digital pin 29

/***************************** Sensor Objects ***********************************/

Adafruit_VL53L0X loxFarRight = Adafruit_VL53L0X();

Adafruit_VL53L0X loxMidRight = Adafruit_VL53L0X();

Adafruit_VL53L0X loxMidLeft = Adafruit_VL53L0X();

Adafruit_VL53L0X loxFarLeft = Adafruit_VL53L0X();

IRrecv irrecv(receiver);

/***************************** Global Variables *********************************/

enum SystemState {

OFF,

STANDBY,

MOVING,

SEARCHING

};

SystemState currentState = OFF;

float yaw = 0;                // Current heading from gyro

float gyroZOffset = 0;        // Gyro calibration offset

unsigned long prevTime = 0;   // For gyro timing

double ipr = 18.85;           // Inches per revolution (from your comment)

double tpr = 900;             // Ticks per revolution (from your comment)

int loudNoiseThreshold = 400; // Sound detection threshold

const float TARGET_DISTANCE = 10.0 * 304.8; // 5ft in mm

const float wheelbase = 13.7; // inches

float vL = 0;

float vR = 0;

volatile int leftencoderpulse = 0;

volatile int rightencoderpulse = 0;

static uint32_t last_decodedRawData = 0;

const unsigned long GYRO_CALIBRATION_TIME = 2000;

const unsigned long SEARCH_STABILIZE_TIME = 2000;

const unsigned long SEARCH_DURATION = 10000;

bool initialCalibrationDone = false;

static unsigned long lastDisturbanceTime = 0;

const unsigned long debounceTime = 500; // 0.5s debounce

bool beepActive = false;          // Control flag for beep mode

unsigned long previousBeepMillis = 0;

const long beepInterval = 500;    // 500ms beep toggle interval

float error = 0, eI = 0, eD = 0, prev_error = 0;

const float Kp = 1.2, Ki = 0.3, Kd = 0;

float omega = 0;

int targetangle = 0;

void computePID() {

error = targetangle - yaw;

eI += error;

eD = error - prev_error;

prev_error = error;

// Anti-windup

eI = constrain(eI, -200, 200);

omega = Kp*error + Ki*eI + Kd*eD;

omega = constrain(omega, -100, 100);

}

float accumulatedDistance = 0;

bool obstacleDetected = false;

unsigned long searchPhaseStart = 0;

bool disturbanceDetected = false;

int flashState = 0;

/***************************** LED Control *************************************/

void setLedState(SystemState state) {

// No change if already in correct state

static SystemState lastState = OFF;

if (state == lastState) return;

lastState = state;

// Turn all off first

digitalWrite(RED, LOW);

digitalWrite(GREEN, LOW);

digitalWrite(BLUE, LOW);

switch(state) {

case OFF:

digitalWrite(RED, HIGH);

break;

case STANDBY:

digitalWrite(RED, HIGH);

digitalWrite(GREEN, HIGH);

break;

case MOVING:

digitalWrite(GREEN, HIGH);

break;

case SEARCHING:

digitalWrite(BLUE, HIGH);

break;

}

}

/***************************** Gyro Functions **********************************/

int16_t readGyroZ() {

Wire.beginTransmission(0x68);

Wire.write(0x47);

Wire.endTransmission(false);

Wire.requestFrom(0x68, 2, true);

return Wire.read() << 8 | Wire.read();

}

void updateGyro() {

int16_t rawGyroZ = readGyroZ();

float gyroZ = (rawGyroZ - gyroZOffset) / 131.0; // ±250dps range

unsigned long currentTime = micros();

float dt = (currentTime - prevTime) / 1000000.0;

prevTime = currentTime;

yaw += gyroZ * dt;

}

void calibrateGyro() {

Serial.println("Calibrating gyro...");

long total = 0;

for(int i = 0; i < 1000; i++) {

total += readGyroZ();

delay(1);

}

gyroZOffset = total / 1000.0;

Serial.print("Gyro offset: ");

Serial.println(gyroZOffset);

}

void setup() {

Serial.begin(115200);

Wire.begin();

// Initialize motor control pins

pinMode(leftdirpin, OUTPUT);

pinMode(rightdirpin, OUTPUT);

pinMode(leftpwmpin, OUTPUT);

pinMode(rightpwmpin, OUTPUT);

analogWrite(leftpwmpin, 0);

analogWrite(rightpwmpin, 0);

// Initialize encoder interrupts

pinMode(leftencoderApin, INPUT);

pinMode(leftencoderBpin, INPUT);

pinMode(rightencoderApin, INPUT);

pinMode(rightencoderBpin, INPUT);

attachInterrupt(digitalPinToInterrupt(leftencoderApin), ALeadingLeft, RISING);

attachInterrupt(digitalPinToInterrupt(leftencoderBpin), BLeadingLeft, RISING);

attachInterrupt(digitalPinToInterrupt(rightencoderApin), ALeadingRight, RISING);

attachInterrupt(digitalPinToInterrupt(rightencoderBpin), BLeadingRight, RISING);

// Initialize LEDs

pinMode(RED, OUTPUT);

pinMode(GREEN, OUTPUT);

pinMode(BLUE, OUTPUT);

// Initialize PIR sensors

pinMode(pirPin1, INPUT);

pinMode(pirPin2, INPUT);

pinMode(pirPin3, INPUT);

pinMode(pirPin4, INPUT);

// Initialize buzzer

pinMode(buzzerPin, OUTPUT);

digitalWrite(buzzerPin, LOW); // Ensure buzzer starts off

// Initialize ToF sensors

pinMode(XSHUT_FAR_RIGHT, OUTPUT);

pinMode(XSHUT_MID_RIGHT, OUTPUT);

pinMode(XSHUT_MID_LEFT, OUTPUT);

pinMode(XSHUT_FAR_LEFT, OUTPUT);

initializeSensor(loxFarRight, XSHUT_FAR_RIGHT, 0x30, "Far Right");

initializeSensor(loxMidRight, XSHUT_MID_RIGHT, 0x31, "Middle Right");

initializeSensor(loxMidLeft, XSHUT_MID_LEFT, 0x32, "Middle Left");

initializeSensor(loxFarLeft, XSHUT_FAR_LEFT, 0x33, "Far Left");

// Initialize IR receiver

irrecv.enableIRIn();

// Initialize MPU

Wire.beginTransmission(0x68);

Wire.write(0x6B); // PWR_MGMT_1

Wire.write(0);    // Wake up

Wire.endTransmission();

// Calibrate sensors

calibrateGyro();

calibrateSensor();

// Initial state

currentState = OFF;

setLedState(OFF);

Serial.println("System initialization complete!");

delay(2000); // Initial stabilization

}

/***************************** Main Loop ***************************************/

void loop() {

// Update sensor data

updateGyro();

// Handle IR input

if (irrecv.decode()) {

translateIR();

irrecv.resume();

}

// State machine processing

processStateMachine();

// Optional: Add periodic debug prints if needed

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 1000) {

Serial.print("State: ");

Serial.print(currentState);

Serial.print(" | Distance: ");

Serial.print(accumulatedDistance);

Serial.print(" | Yaw: ");

Serial.println(yaw);

lastPrint = millis();

}

}

/***************************** ToF Sensor Initialization ***********************/

void initializeSensor(Adafruit_VL53L0X &sensor, int xshutPin, uint8_t newAddress, const char* position) {

digitalWrite(xshutPin, LOW);

delay(10);

digitalWrite(xshutPin, HIGH);

delay(10);

if (!sensor.begin(newAddress)) {

Serial.print(position);

Serial.println(" sensor failed to boot!");

while(1);

}

Serial.print(position);

Serial.print(" sensor initialized at 0x");

Serial.println(newAddress, HEX);

}

/***************************** Modified Core Functions **********************************/

void processStateMachine() {

static SystemState previousState = OFF;

// Only process if state changed

if (currentState != previousState) {

previousState = currentState;

switch (currentState) {

case STANDBY:

handleStandby();

break;

case MOVING:

handleMoving();

break;

case SEARCHING:

handleSearching();

break;

case OFF:

handleOff();

break;

}

}

// Continuous processing for certain states

if (currentState == MOVING) {

// Obstacle detection and movement

if (checkFrontObstacle()) {

handleObstacle();

} else {

moveForward();

updateDistance();

if (accumulatedDistance >= TARGET_DISTANCE) {

transitionToSearching();

}

}

}

else if (currentState == SEARCHING) {

// Search mode processing

if (checkDisturbances()) {

stopMotors();

disturbanceDetected = true;

flashSearchLights();

}

if (millis() - searchPhaseStart >= SEARCH_DURATION) {

transitionToMoving();

}

}

}

void handleStandby() {

setLedState(STANDBY);

stopMotors();

// Wait for IR command to transition to moving

}

void handleMoving() {

static unsigned long lastObstacleTime = 0;

const unsigned long obstacleCooldown = 1000; // 1 second cooldown

setLedState(MOVING);

// Initialization on state entry

if (!initialCalibrationDone) {

calibrateGyro();

initialCalibrationDone = true;

resetEncoders();

accumulatedDistance = 0;

Serial.println("Starting movement to 5ft");

}

// Check for obstacles with cooldown

if (millis() - lastObstacleTime > obstacleCooldown && checkFrontObstacle()) {

handleObstacle();

lastObstacleTime = millis();

return;

}

// Normal movement

moveForward();

updateDistance();

// Check completion

if (accumulatedDistance >= TARGET_DISTANCE) {

Serial.println("Reached 5ft target");

transitionToSearching();

initialCalibrationDone = false;

}

}

void handleSearching() {

static bool stabilizationDone = false;

// State entry initialization

if (!stabilizationDone) {

setLedState(SEARCHING);

stopMotors();

searchPhaseStart = millis();

stabilizationDone = true;

disturbanceDetected = false;

digitalWrite(BLUE, HIGH);

digitalWrite(buzzerPin, LOW); // Ensure buzzer starts quiet

delay(SEARCH_STABILIZE_TIME); // Sensor stabilization

}

// Check sensors

if (checkDisturbances()) {

flashSearchLights(); // This now handles both LEDs and buzzer

} else {

// Ensure buzzer is off when no disturbance

digitalWrite(buzzerPin, LOW);

}

// Time-based state exit

if (millis() - searchPhaseStart >= SEARCH_DURATION) {

stabilizationDone = false;

transitionToMoving();

}

}

/***************************** New Helper Functions **********************************/

bool checkFrontObstacle() {

VL53L0X_RangingMeasurementData_t measure;

loxMidLeft.rangingTest(&measure, false);

return (measure.RangeMilliMeter <= 610 && measure.RangeStatus != 4); // 2ft = ~610mm

}

void handleObstacle() {

stopMotors();

delay(500); // Pause before turning

// Random turn (100-280 degrees) with direction based on sensor reading

int turnAngle = random(100, 280);

bool turnRight = random(0, 2); // Random direction

Serial.print("Obstacle! Turning ");

Serial.print(turnAngle);

Serial.println(turnRight ? "° right" : "° left");

if (turnRight) {

performGyroTurn(turnAngle);

} else {

performGyroTurn(-turnAngle); // Negative for left turn

}

// Resume movement

moveForward();

}

void performGyroTurn(float degrees) {

float targetYaw = yaw + degrees;

float startYaw = yaw;

vL = 210-((omega * wheelbase) / 2)*0.152;

vR = 255-((omega * wheelbase) / 2)*0.152;

// Determine turn direction

bool turnRight = degrees > 0;

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, HIGH);

// Apply turning power

analogWrite(leftpwmpin, vL);

analogWrite(rightpwmpin, vR);

// Wait until target reached

while (abs(yaw - targetYaw) > 2.0) {

updateGyro();

Serial.println(yaw);

delay(10);

}

stopMotors();

Serial.print("Turn completed. Final yaw: ");

Serial.println(yaw);

}

void updateDistance() {

// Convert encoder ticks to distance (mm)

float leftDistance = (leftencoderpulse / tpr) * ipr * 25.4;

float rightDistance = (rightencoderpulse / tpr) * ipr * 25.4;

accumulatedDistance = (leftDistance + rightDistance) / 2.0; // Average of both wheels

Serial.print("Distance: ");

Serial.print(accumulatedDistance / 304.8, 2); // Display in feet with 2 decimals

Serial.println(" ft");

}

void transitionToSearching() {

digitalWrite(buzzerPin, LOW); // Ensure buzzer off when entering search

currentState = SEARCHING;

searchPhaseStart = millis();

setLedState(SEARCHING); // Explicitly set LED state

Serial.println("Transitioning to Search Mode");

}

void transitionToMoving() {

digitalWrite(buzzerPin, LOW); // Ensure buzzer off when leaving search

currentState = MOVING;

accumulatedDistance = 0;

resetEncoders();

setLedState(MOVING); // Explicitly set LED state

Serial.println("Resuming Movement");

}

void flashSearchLights() {

static unsigned long lastFlashTime = 0;

if (millis() - lastFlashTime >= beepInterval) {

lastFlashTime = millis();

// Toggle both LEDs and buzzer together

flashState = !flashState;

if (flashState) {

// Purple (Red + Blue) + Buzzer ON

digitalWrite(RED, HIGH);

digitalWrite(GREEN, LOW);

digitalWrite(BLUE, HIGH);

digitalWrite(buzzerPin, HIGH);

} else {

// Cyan (Green + Blue) + Buzzer OFF

digitalWrite(RED, LOW);

digitalWrite(GREEN, HIGH);

digitalWrite(BLUE, HIGH);

digitalWrite(buzzerPin, LOW);

}

}

}

/***************************** Modified IR Functions **********************************/

void translateIR() {

if (irrecv.decode()) {

static unsigned long lastIRTime = 0;

unsigned long currentTime = millis();

// Debounce - ignore repeats within 200ms

if (currentTime - lastIRTime > 200) {

lastIRTime = currentTime;

switch(irrecv.decodedIRData.decodedRawData) {

case 0xBA45FF00: // POWER

if (currentState == OFF) {

currentState = STANDBY;  // OFF → STANDBY

} else if (currentState == STANDBY) {

currentState = MOVING;   // STANDBY → MOVING

} else {

currentState = OFF;      // MOVING/SEARCHING → OFF

}

setLedState(currentState);

break;

case 0xB847FF00: // RESET

currentState = STANDBY;    // Force reset to STANDBY

accumulatedDistance = 0;

resetEncoders();

setLedState(currentState);

break;

}

}

irrecv.resume();

}

}

/***************************** Motor Control Enhancements **********************************/

void moveForward() {

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 110);

analogWrite(rightpwmpin, 200);

}

void resetEncoders() {

leftencoderpulse = 0;

rightencoderpulse = 0;

}

void ALeadingLeft() {

if(digitalRead(leftencoderBpin) == LOW) leftencoderpulse--;

else leftencoderpulse++;

}

void BLeadingLeft() {

if(digitalRead(leftencoderApin) == LOW) leftencoderpulse++;

else leftencoderpulse--;

}

void ALeadingRight() {

if(digitalRead(rightencoderBpin) == LOW) rightencoderpulse++;

else rightencoderpulse--;

}

void BLeadingRight() {

if(digitalRead(rightencoderApin) == LOW) rightencoderpulse--;

else rightencoderpulse++;

}

void stopMotors() {

digitalWrite(leftdirpin, HIGH);

digitalWrite(rightdirpin, LOW);

analogWrite(leftpwmpin, 255);

analogWrite(rightpwmpin, 0);

}

void handleOff() {

digitalWrite(RED, HIGH);

digitalWrite(GREEN, LOW);

digitalWrite(BLUE, LOW);

stopMotors();

}

/***************************** Sensor Functions **********************************/

bool checkDisturbances() {

// Check PIR sensors

bool motionDetected = digitalRead(pirPin1) || digitalRead(pirPin2) || digitalRead(pirPin3) || digitalRead(pirPin4);

// Check sound with threshold

bool soundDetected = analogRead(sensorAnalogPin) > loudNoiseThreshold;

if ((motionDetected || soundDetected) &&

(millis() - lastDisturbanceTime > debounceTime)) {

lastDisturbanceTime = millis();

return true;

}

return false;

}

void calibrateSensor() {

long sum = 0;

for(int i = 0; i < 50; i++) {

sum += analogRead(sensorAnalogPin);

delay(20);

}

loudNoiseThreshold = (sum / 50) + 400;

Serial.print("Sound threshold: ");

Serial.println(loudNoiseThreshold);

}

void resetSystem() {

currentState = OFF;

accumulatedDistance = 0;

yaw = 0;

leftencoderpulse = 0;

rightencoderpulse = 0;

stopMotors();

setLedState(OFF);

}